Xpeech CDR Report Explanation

92.8.08

## Index

Index	1

Basic rules:	1

Events table	2

ISDN PRI billing analyses	3

Example 1:	3

Example 2:	3

## Basic rules:

- Every record consist in a single line. By using “\r\n” (0x0D, 0x0A) 2 byte as end of line.

- Every record that contain more than one item, then it will be separated by space.

- Possible content types are:

  - Date: YYYY/MM/DD format, YYYY for 4 digits of year, MM for 2 digits of month, DD for 2 digits of day. If the digits is less than needed, zero will be placed in front, example: 0901/07/01。

  - Time: hh:mm:ss format, hh for 2 digits of 24hr format hour, mm for 2 digits of minutes, ss for 2 digits of seconds.

  - Integer

  - Command: Command is combined with English character with “_”, example: HOOK_OFF. Also could be a single “*” sign.

  - String: String will be with in medium brackets “[” and “]”, if the string is empty, it will be represent like “[ ]”.

- Every 1st and 2nd item of record is server date and time. This represent the time server log this record.

- Every 3rd item of record is command. According to different command will have different command string.

- Every 4th item of record is Call-Ref. This is a serial number that generated by Gateway itself. If you want to search value of a call, please use Call-Ref to find the correspond command. Example: by searching at HOOK_OFF or CALL_IN, you could see which port was used in the Gateway.

- Every 5th and 6th item of record is Gateway local date and time. Actual calling time is more accurate in this item. Time in Server is just for reference only.

- There might be more item added according to command needs.

## Events table

| * | Please ignore this record |  |
| --- | --- | --- |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | * | command |
| 4 | Ignore this string |  |
| Example: 2003/07/23 11:21:31 * INFO Hello |  |  |
| Explanation: This record is not CDR, can be ignored. |  |  |
| #### HOOK_OFF | Gateway hook-off Port-No |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | #### HOOK_OFF | command |
| 4 | Call-Ref | integer |
| 5 | Gateway-date | date |
| 6 | Gateway-time | time |
| 7 | Port-No | integer |
| Ex: 2003/07/23 11:21:31 HOOK_OFF 123 2003/07/23 11:21:30 3 |  |  |
| Exp: Call-Ref(123) Gateway 3rd port hook-off |  |  |
| ##### DIAL | VoIP Dialed number |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | #### DIAL |  |
| 4 | Call-Ref | integer |
| 5 | Gateway-date | date |
| 6 | Gateway-time | time |
| 7 | Dialed out | string |
| Ex: 2003/07/23 11:12:25 DIAL 123 2003/07/23 11:12:25 [22520199] |  |  |
| Exp: Call-Ref(123) dialed 22520199。 |  |  |
| ##### CALL_IN | VoIP call in to Gateway Caller serial number is Peer-Call-Ref Caller’s ID is Caller-ID Caller dial out number is Dialed-Number(Note: Might be empty, example: 2 stage dialing.) |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | #### CALL_IN | command |
| 4 | Call-Ref | integer |
| 5 | Gateway-date | date |
| 6 | Gateway-time | time |
| 7 | Peer-Call-Ref | integer |
| 8 | Caller-ID | string |
| 9 | Dialed-Number | string |
| Ex: 2003/07/23 11:12:25 CALL_IN 123 2003/07/23 11:12:25 39724 [22520199] [117] |  |  |
| Exp: Call-Ref(123) VoIP call in Gateway, caller serial is 39724, caller-ID is 22520199, caller wants to dial out 117 from Gateway |  |  |
| ##### USES_PORT | Find Gateway idle Port-No |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | Port-No | integer |
| Ex: 2003/07/23 11:12:25 USES_PORT 2003/07/23 11:12:25 1 |  |  |
| Exp: Call-Ref(123) VoIP call-in Gateway and detect 1st line is idle or assign 1st port. |  |  |
| ##### CALL_IN_REJECT | VoIP reject call in with Reject-Reason |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | #### CALL_IN_REJECT | command |
| 4 | Call-Ref | integer |
| 5 | Gateway-date | date |
| 6 | Gateway-time | time |
| 7 | Reject-Reason | integer |
| Ex: 2003/07/23 11:12:25 CALL_IN_REJECT 123 2003/07/23 11:12:25 13 |  |  |
| Exp: Call-Ref(123) with reason of (13) to reject VoIP call in. |  |  |
| ##### CALL_OUT_FAIL | VoIP call out fail with Fail-Reason |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | #### CALL_OUT_FAIL | command |
| 4 | Call-Ref | integer |
| 5 | Gateway-date | date |
| 6 | Gateway-time | time |
| 7 | Fail-Reason | integer |
| Ex: 2003/07/23 11:12:25 CALL_OUT_FAIL 123 2003/07/23 11:12:25 7 |  |  |
| Exp: Call-Ref(123) with reason (7) of VoIP call out fail. |  |  |
| ##### RING | Ring |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | #### RING | command |
| 4 | Call-Ref | integer |
| 5 | Gateway-date | date |
| 6 | Gateway-time | time |
| Ex: 2003/07/23 11:12:25 RING 123 2003/07/23 11:12:25 |  |  |
| Exp: Call-Ref(123) Gateway ring. |  |  |
| ##### RING_BACK | VoIP get ring back tone |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | #### RING_BACK | command |
| 4 | Call-Ref | integer |
| 5 | Gateway-date | date |
| 6 | Gateway-time | time |
| Ex: 2003/07/23 11:12:25 RING_BACK 123 2003/07/23 11:12:25 |  |  |
| Exp: Call-Ref(123) got ring back tone |  |  |
| ##### TALK | Set voice type: Codec type is Coder, codec mode is Talk-Mode (0 is silent, 1 is talk do not listen, 16 is listen do not talk, 17 is full duplex.) If Coder and Talk-Mode are 0, it means voice off. |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | #### TALK | command |
| 4 | Call-Ref | integer |
| 5 | Gateway-date | date |
| 6 | Gateway-time | time |
| 7 | Coder | integer |
| 8 | Talk-Mode | integer |
| Ex: 2003/07/23 11:12:25 TALK 123 2003/07/23 11:12:25 18 17 |  |  |
| Exp: Call-Ref(123) start talking with Coder(18) and Talk-Mode(17 = full duplex) |  |  |
| ##### FAX_DETECTED | Detected FAX |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | #### FAX_DETECTED | command |
| 4 | Call-Ref | integer |
| 5 | Gateway-date | date |
| 6 | Gateway-time | time |
| Ex: 2003/07/23 11:12:25 FAX_DETECTED 123 2003/07/23 11:12:25 |  |  |
| Exp: Call-Ref(123) FAX signal detected. |  |  |
| ##### HOOK_ON | Hook on this call |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | #### HOOK_ON | command |
| 4 | Call-Ref | integer |
| 5 | Gateway-date | date |
| 6 | Gateway-time | time |
| Ex: 2003/07/23 11:12:25 HOOK_ON 123 2003/07/23 11:12:25 |  |  |
| Exp: Call-Ref(123) hook on the call |  |  |
| ##### REMOTE_HOOK_ON | Remote side hook on |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | #### REMOTE_HOOK_ON | command |
| 4 | Call-Ref | integer |
| 5 | Gateway-date | date |
| 6 | Gateway-time | time |
| Ex: 2003/07/23 11:12:25 REMOTE_HOOK_ON 123 2003/07/23 11:12:25 |  |  |
| Exp: Call-Ref(123) remote side hook on. |  |  |
| ##### PSTN_HOOK_OFF | PSTN line is hooked off |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | #### PSTN_HOOK_OFF | command |
| 4 | Call-Ref | integer |
| 5 | Gateway-date | date |
| 6 | Gateway-time | time |
| 7 | Pstn-Port-No | integer |
| Ex: 2003/07/23 11:12:25 PSTN_HOOK_OFF 123 2003/07/23 11:12:25 2 |  |  |
| Exp: Call-Ref(123) 2nd line of PSTN is hook-off |  |  |
| ##### PSTN_DIAL | PSTN dial out number (Dialed-Number) |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | #### PSTN_DIAL | command |
| 4 | Call-Ref | integer |
| 5 | Gateway-date | date |
| 6 | Gateway-time | time |
| 7 | Dialed-Number | string |
| Ex: 2003/07/23 11:12:25 PSTN_DIAL 123 2003/07/23 11:12:25 [117] |  |  |
| Exp: Call-Ref(123) dialed 117 on PSTN line. |  |  |
| ##### PSTN_RING_BACK | Ring back from PSTN |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | #### PSTN_RING_BACK | command |
| 4 | Call-Ref | integer |
| 5 | Gateway-date | date |
| 6 | Gateway-time | time |
| Ex: 2003/07/23 11:12:25 PSTN_RING_BACK 123 2003/07/23 11:12:25 |  |  |
| Exp: Call-Ref(123) ring back from PSTN. |  |  |
| ##### PSTN_CONNECTED | PSTN connected |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | #### PSTN_CONNECTED | command |
| 4 | Call-Ref | integer |
| 5 | Gateway-date | date |
| 6 | Gateway-time | time |
| Ex: 2003/07/23 11:12:25 PSTN_CONNECTED 123 2003/07/23 11:12:25 |  |  |
| Exp: Call-Ref(123) PSTN connected. |  |  |
| ##### PSTN_CALL_OUT_FAIL | PSTN call out failed with Fail-Reason |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | #### PSTN_CALL_OUT_FAIL | command |
| 4 | Call-Ref | integer |
| 5 | Gateway-date | date |
| 6 | Gateway-time | time |
| 7 | Fail-Reason | integer |
| Ex: 2003/07/23 11:12:25 PSTN_CALL_OUT_FAIL 123 2003/07/23 11:12:25 2 |  |  |
| Exp: Call-Ref(123) PSTN call out failed with Fail-Reason (2) |  |  |
| ##### PSTN_HOOK_ON | PSTN hook on |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | #### PSTN_HOOK_ON | command |
| 4 | Call-Ref | integer |
| 5 | Gateway-date | date |
| 6 | Gateway-time | time |
| Ex: 2003/07/23 11:12:25 PSTN_HOOK_ON 123 2003/07/23 11:12:25 |  |  |
| Exp: Call-Ref(123) PSTN hook on. |  |  |
| ##### PSTN_CALL_IN | Call in from PSTN, serial number of PSTN is PSTN-Call-Ref(might be meaningless), Caller number is Caller-ID, use PSTN-Port-No |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | #### PSTN_CALL_IN | command |
| 4 | Call-Ref | integer |
| 5 | Gateway-date | date |
| 6 | Gateway-time | time |
| 7 | PSTN-Call-Ref | integer |
| 8 | Caller-ID | string |
| 9 | Pstn-Port-No | integer |
| Ex: 2003/07/23 11:12:25 PSTN_CALL_IN 123 2003/07/23 11:12:25 2001 [22520199] 12 |  |  |
| Exp: Call-Ref(123) call in from PSTN with serial number of 2001. Caller-ID is 22520199 and used 12th line. |  |  |
| PSTN_REMOTE_HOOK_ON | Remote PSTN side hook on |  |
| 1 | Server-date | date |
| 2 | Server-time | time |
| 3 | PSTN_REMOTE_HOOK_ON | command |
| 4 | Call-Ref | integer |
| 5 | Gateway-date | date |
| 6 | Gateway-time | time |
| Ex: 2003/07/23 11:12:25 PSTN_REMOTE_HOOK_ON 123 2003/07/23 11:12:25 |  |  |
| Exp: Call-Ref(123) remote PSTN side hook on. |  |  |

## ISDN PRI billing analyses

- Ignore all “*” item.

- Some one call in will have CALL_IN event. Remember it’s Port-No(line number), Call-Ref(serial number), Peer-Call-Ref(remote side serial number), Caller-ID, Dialed-Number(PSTN dial out number, might be empty cause by 2nd stage dialing) for reference.

- Later on will have PSTN_DIAL even, please record this Call-Ref Dialed-Number(This is real dial out number)。

- If dial out OK, will receive PSTN_CONNECTED, means start talking. Only when receive this message means successful dial out.

- If remote side hook on, will have event REMOTE_HOOK_ON, but real PSTN on hook time is at PSTN_HOOK_ON.

- If PSTN hook on first, will have event PSTN_REMOTE_HOOK_ON, this will also be the real PSTN hook on time. (next PSTN_HOOK_ON can be ignored)

- Conclusion, PSTN call starts at PSTN_CONNECTED, ends at PSTN_HOOK_ON(remote hook on first) or PSTN_REMOTE_HOOK_ON(PSTN hook on first), these event depends on which starts first.

- VoIP call starts at CALL_IN event, ends at REMOTE_HOOK_ON or HOOK_ON event, no matter successful or not. (This assume that you don’t care about actual conversation time)

- Others can be ignored.

## Example 1:

Called by VOIP, off-net call to 22520199 through ISDN_PRI. The Call detail from 2003/07/24 17:00:56 to 2003/07/24 17:01:07. On-hook by PSTN.

2003/07/24 17:00:31 * LOGIN [ISDN61_8888_8] 2003/07/24 17:00:30

2003/07/24 17:00:42 * INFO 2003/07/24 17:00:41 [<<< ISDN ON-LINE >>>]

2003/07/24 17:00:52 CALL_IN 1 2003/07/24 17:00:51 12345 [54321] []

2003/07/24 17:00:52 USES_PORT 2003/07/24 17:00:52 25

2003/07/24 17:00:52 HOOK_OFF 1 2003/07/24 17:00:51 25

2003/07/24 17:00:56 TALK 1 2003/07/24 17:00:55 4 17

2003/07/24 17:00:56 PSTN_HOOK_OFF 1 2003/07/24 17:00:55 25 24

2003/07/24 17:00:56 PSTN_DIAL 1 2003/07/24 17:00:55 [22520199]

2003/07/24 17:00:57 PSTN_RING_BACK 1 2003/07/24 17:00:55

2003/07/24 17:00:57 PSTN_CONNECTED 1 2003/07/24 17:00:56

2003/07/24 17:01:08 REMOTE_HOOK_ON 1 2003/07/24 17:01:07

2003/07/24 17:01:08 PSTN_HOOK_ON 1 2003/07/24 17:01:07

2003/07/24 17:01:08 HOOK_ON 1 2003/07/24 17:01:07

2003/07/24 17:01:08 TALK 1 2003/07/24 17:01:07 0 0

## Example 2:

Called by VOIP, off-net call to 117 through ISDN_PRI. The Call detail from 2003/07/24:17:01:27 to 2003/07/24 17:01:40. On-hook by VOIP.

2003/07/24 17:01:27 CALL_IN 2 2003/07/24 17:01:26 12345 [54321] []

2003/07/24 17:01:27 USES_PORT 2003/07/24 17:01:26 23

2003/07/24 17:01:27 HOOK_OFF 2 2003/07/24 17:01:26 23

2003/07/24 17:01:28 TALK 2 2003/07/24 17:01:27 4 17

2003/07/24 17:01:28 PSTN_HOOK_OFF 2 2003/07/24 17:01:27 23 22

2003/07/24 17:01:28 PSTN_DIAL 2 2003/07/24 17:01:27 [117]

2003/07/24 17:01:28 PSTN_RING_BACK 2 2003/07/24 17:01:27

2003/07/24 17:01:28 PSTN_CONNECTED 2 2003/07/24 17:01:27

2003/07/24 17:01:41 REMOTE_HOOK_ON 2 2003/07/24 17:01:40

2003/07/24 17:01:41 PSTN_HOOK_ON 2 2003/07/24 17:01:40

2003/07/24 17:01:41 HOOK_ON 2 2003/07/24 17:01:40

2003/07/24 17:01:42 TALK 2 2003/07/24 17:01:40 0 0